Flash    DTC      SPN-FMI    DIAGNOSTIC CODE DESCRIPTION

00       84-14      84-14     QUICKSTOP OCCURRENCE

00       52-11      52-11     INLET AIR SHUTOFF SHUTDOWN

00      166-14     166-14     RATED ENGINE POWER SPECIAL INSTRUCTIONS

00      246-11     597-11     BRAKE PEDAL SWITCH #1 FAULT

00      247-11     603-11     BRAKE PEDAL SWITCH #2 FAULT

00      171-03     171-03     OUTSIDE AIR TEMP SENSOR VOLTAGE HIGH

00      171-04     171-04     OUTSIDE AIR TEMP SENSOR VOLTAGE LOW

00      171-11     171-11     OUTSIDE AIR TEMP DATA LOST

01       71-00      71-00     IDLE SHUTDOWN OVERRIDE

12      111-02     111-02     INVALID COOLANT LEVEL

12      111-03     111-03     COOLANT LEVEL SENSOR VOLTAGE HIGH

12      111-04     111-04     COOLANT LEVEL SENSOR VOLTAGE LOW

13      174-03     174-03     FUEL TEMP VOLTAGE HIGH

13      174-03     174-04     FUEL TEMP VOLTAGE LOW

13      224-11                THEFT DETERRENT ACTIVE

13      224-14                THEFT DETERRENT ACTIVE WITH ENGINE CRANKING

14      121-05    1072-05     RETARDER SOLENOID LOW/HIGH VOLTAGE LOW

14      121-06    1072-06     RETARDER SOLENOID LOW/HIGH VOLTAGE HIGH

14      122-05    1073-05     RETARDER SOLENOID MED/HIGH VOLTAGE LOW

14      122-06    1073-06     RETARDER SOLENOID MED/HIGH VOLTAGE HIGH

21      232-03     620-03     5 VOLT SUPPLY VOLTAGE HIGH

21      232-04     620-04     5 VOLT SUPPLY VOLTAGE LOW

21       41-03     678-03     8 VOLT SUPPLY VOLTAGE HIGH

21       41-04     678-04     8 VOLT SUPPLY VOLTAGE LOW

24      100-03     100-03     OIL PRESSURE SENSOR VOLTAGE HIGH

24      100-04     100-04     OIL PRESSURE SENSOR VOLTAGE LOW

25      102-03     102-03     BOOST PRESSURE SENSOR VOLTAGE HIGH

25      102-04     102-04     BOOST PRESSURE SENSOR VOLTAGE LOW

25      102-07     102-07     BOOST PRESSURE NOT RESPONDING

26      108-03     108-03     BAROMETRIC PRESSURE VOLTAGE HIGH

26      108-04     108-04     BAROMETRIC PRESSURE VOLTAGE LOW

26      108-10     108-10     BAROMETRIC PRESSURE SIGNAL RATE OF CHANGE

27      110-03     110-03     COOLANT TEMPERATURE SENSOR VOLTAGE HIGH

27      110-04     110-04     COOLANT TEMPERATURE SENSOR VOLTAGE LOW

28       91-13      91-13     ACCELERATOR PEDAL POSITION OUT OF CALIBRATION

29       30-08     644-08     PTO THROTTLE SIGNAL INVALID

29       30-13     644-13     PTO THROTTLE SIGNAL CALIBRATION

31       84-01      84-01     LOSS OF WHEEL-BASED VEHICLE SPEED SIGNAL

32       91-08      91-08     ACCELERATOR PEDAL POSITION INVALID

34      190-08     190-08     PRIMARY ENGINE SPEED LOSS OF SIGNAL

34       64-08     723-08     SECONDARY ENGINE SPEED LOSS OF SIGNAL

35      190-00     190-00     ENGINE OVERSPEED WARNING

36       84-02      84-02     VEHICLE SPEED SIGNAL INVALID

36       84-08      84-08     VEHICLE SPEED SIGNAL OUT OF RANGE

36       84-10      84-10     VEHICLE SPEED SIGNAL RATE OF CHANGE

38      105-03     105-03     INTAKE MANIFOLD AIR TEMPERATURE SENSOR VOLTAGE HIGH

38      105-04     105-03     INTAKE MANIFOLD AIR TEMPERATURE SENSOR VOLTAGE LOW

41       84-00      84-00     VEHICLE OVERSPEED WARNING

42       22-11     637-11     PRIMARY TO SECONDARY ENGINE SPEED SIGNAL CALIBRATION

42       22-13     637-13     ENGINE SPEED SIGNAL CALIBRATION NOT PERFORMED

46      100-01     100-01     LOW OIL PRESSURE WARNING

46      100-11     100-11     VERY LOW OIL PRESSURE 46

47       71-01                IDLE SHUTDOWN OCCURRENCE

47       71-14                PTO SHUTDOWN OCCURRENCE

51      168-00     168-00     EXCESSIVE ECM BATTERY POWER

51      168-01     168-01     LOW ECM BATTERY POWER

51      168-02     168-02     INTERMITTENT BATTERY POWER TO THE ECM

55                            NO DIAGNOSTIC CODE DETECTED

56      253-11     630-11     CHECK TRANSMISSION CUSTOMER PARAMETERS

56      253-02     630-02     CHECK CUSTOMER OR SYSTEM PARAMETERS

58      231-02     639-02     J1939 DATA INCORRECT

58      231-12     639-12     J1939 DEVICE NOT RESPONDING

59      252-11     631-11     ENGINE SOFTWARE INCORRECT

61      110-00     110-01     HIGH COOLANT TEMPERATURE WARNING

61      110-11     110-11     VERY HIGH COOLANT TEMPERATURE

62      111-01     111-01     LOW COOLANT LEVEL WARNING

62      111-11     111-11     VERY LOW COOLANT LEVEL

62      111-14     111-14     LOW COOLANT LEVEL WARNING (LESS THAN 6 ENGINE HOURS)

64      105-00     105-00     HIGH INTAKE MANIFOLD AIR TEMPERATURE WARNING

64      105-11     105-11     VERY HIGH INTAKE MANIFOLD AIR TEMPERATURE WARNING

65      174-00     174-00     HIGH FUEL TEMPERATURE WARNING

66       54-05     706-05     AUXILIARY OUTPUT #06 CURRENT LOW

66       54-06     706-06     AUXILIARY OUTPUT #06 CURRENT HIGH

67       55-05     707-05     AUXILIARY OUTPUT #07 CURRENT LOW

67       55-06     707-06     AUXILIARY OUTPUT #07 CURRENT HIGH

68      191-07                TRANSMISSION NOT RESPONDING

71       43-02                IGNITION KEY SWITCH FAULT 71

72        1-05     651-05     CYLINDER #1 INJECTOR CURRENT LOW

72        1-06     651-06     CYLINDER #1 INJECTOR CURRENT HIGH

72        2-05     652-05     CYLINDER #2 INJECTOR CURRENT HIGH

72        2-05     642-06     CYLINDER #2 INJECTOR CURRENT LOW

73        3-05     653-05     CYLINDER #3 INJECTOR CURRENT LOW

73        3-06     653-06     CYLINDER #3 INJECTOR CURRENT HIGH

73        4-05     654-05     CYLINDER #4 INJECTOR CURRENT LOW

73        4-06     654-06     CYLINDER #4 INJECTOR CURRENT HIGH

74        5-05     655-05     CYLINDER #5 INJECTOR CURRENT LOW

74        5-06     655-06     CYLINDER #5 INJECTOR CURRENT HIGH

74        6-05     656-05     CYLINDER #6 INJECTOR CURRENT LOW

74        6-06     656-06     CYLINDER #6 INJECTOR CURRENT HIGH

92      285-05    2950-05     INTAKE VALVE ACTUATOR #1 CURRENT LOW

92      285-06    2950-06     INTAKE VALVE ACTUATOR #1 CURRENT HIGH

92      285-07    2950-07     INTAKE VALVE ACTUATOR #1 NOT RESPONDING

92      286-05    2951-05     INTAKE VALVE ACTUATOR #2 CURRENT LOW

92      286-06    2951-06     INTAKE VALVE ACTUATOR #2 CURRENT HIGH

92      286-07    2951-07     INTAKE VALVE ACTUATOR #2 NOT RESPONDING

93      287-05    2952-05     INTAKE VALVE ACTUATOR #3 CURRENT LOW

93      287-06    2952-06     INTAKE VALVE ACTUATOR #3 CURRENT HIGH

93      287-07    2952-07     INTAKE VALVE ACTUATOR #3 CURRENT NOT RESPONDING

93      288-05    2953-05     INTAKE VALVE ACTUATOR #4 CURRENT LOW

93      288-06    2953-06     INTAKE VALVE ACTUATOR #4 CURRENT HIGH

93      288-07    2953-07     INTAKE VALVE ACTUATOR #4 CURRENT NOT RESPONDING

94      289-05    2954-05     INTAKE VALVE ACTUATOR #5 CURRENT LOW

94      289-06    2954-06     INTAKE VALVE ACTUATOR #5 CURRENT HIGH

94      289-07    2954-07     INTAKE VALVE ACTUATOR #5 CURRENT NOT RESPONDING

94      290-05    2955-05     INTAKE VALVE ACTUATOR #6 CURRENT LOW

94      290-06    2955-06     INTAKE VALVE ACTUATOR #6 CURRENT HIGH

94      290-07    2955-07     INTAKE VALVE ACTUATOR #6 CURRENT NOT RESPONDING

95      385-01    2948-01     LOW INTAKE VALVE ACTUATION SYSTEM OIL PRESSURE

95      385-03    2948-03     INTAKE VALVE ACTUATION OIL PRESSURE VOLTAGE HIGH

95      385-04    2948-04     INTAKE VALVE ACTUATION OIL PRESSURE VOLTAGE LOW

97      283-05    2949-05     INTAKE VALVE ACTUATION SYSTEM OIL PRESSURE SOLENOID CURRENT LOW

97      283-06    2949-06     INTAKE VALVE ACTUATION SYSTEM OIL PRESSURE SOLENOID CURRENT HIGH

97      283-07    2949-07     INTAKE VALVE ACTUATION SYSTEM OIL PRESSURE NOT RESPONDING

98      284-05    2988-05     ENGINE COOLANT DIVERTER SOLENOID CURRENT LOW

98      284-06    2988-06     ENGINE COOLANT DIVERTER SOLENOID CURRENT HIGH